IPsec Lab Cisco Configuration

STEP 1 Configure the IKE Phase 1  Policy (ISAKMP Policy)

Note:  Useful command is:

Router(config)#crypto isakmp ?

aggressive-mode       Disable ISAKMP aggressive mode

client                        Set client configuration policy

enable                       Enable ISAKMP

identity                      Set the identity which ISAKMP will use

invalid-spi-recovery  Initiate IKE and send Invalid SPI Notify

keepalive                   Set a keepalive interval for use with IOS peers

key                             Set pre-shared key for remote peer

nat                              Set a nat  keepalive interval for use with IOS peers

peer                            Set Peer Policy

policy                        Set policy for an ISAKMP protection suite

profile                       Define ISAKMP Profiles

xauth                         Set Extended Authentication values

Cisco literature refers to IKE Phase 1 as the ISAKMP policy.

It is configured using the command:

crypto isakmp policy <priority #>

Multiple policies can be configured and the priority number, which ranges

from 1 to 10,000, denotes the order of preference that a given policy will be

negotiated with an ISAKMP peer.  The  lower value has the  higher priority.

Once in the ISAKMP configuration mode, denoted by the ‘Router(config-isakmp)#’ prompt, the following parameters can be specified are:

Encryption Algorithm

Hash Algorithm

Authentication Method

Group Lifetime

Note: The following command is useful for helping in configuration

Router(config-isakmp)#?

ISAKMP commands:

authentication  Set authentication method for protection suite

default              Set a command to its defaults

encryption       Set encryption algorithm for protection suite

exit                  Exit from ISAKMP protection suite configuration mode

group               Set the Diffie-Hellman group

hash                 Set hash algorithm for protection suite

lifetime            Set lifetime for ISAKMP security association

no              Negate a command or set its defaults

STEP 2  Set the ISAKMP Identity

The ISAKMP identity specifies how the IKE Phase 1 peer is identified,

which can be either by IP address or host name.

The command to use is:

crypto isakmp identity {address | hostname | dn }

By default, a peer’s ISAKMP identity is the peer’s IP address.

If you decide to change the default  just keep in mind that it is

best to always be consistent across your entire IPsec-protected

network in the way you choose to define a peer’s identity.

STEP 3  Configure the IPsec AH and ESP Parameters

Useful command to is :

Router20(config)#crypto ipsec ?

client                        Configure a client

df-bit                         Handling of encapsulated DF bit.

fragmentation           Handling of fragmentation of near-MTU sized packets

nat-transparency      IPsec NAT transparency model

optional                     Enable optional encryption for IPSec

profile                        Configure an ipsec policy profile

security-association  Security association parameters

transform-set             Define transform and settings

The AH and ESP parameters are configured with the following commands:

crypto ipsec transform-set transform-set-name <transform 1> <transform 2>

mode [tunnel | transport]

crypto ipsec security-association lifetime seconds seconds

The following options are possible for the transforms:

Router20(config)#crypto ipsec transform-set test ?

ah-md5-hmac        AH-HMAC-MD5 transform

ah-sha-hmac         AH-HMAC-SHA transform

comp-lzs                IP Compression using the LZS compression algorithm

esp-3des                ESP transform using 3DES(EDE) cipher (168 bits)

esp-aes                  ESP transform using AES cipher

esp-des                  ESP transform using DES cipher (56 bits)

esp-md5-hmac       ESP transform using HMAC-MD5 auth

esp-null                   ESP transform w/o cipher

esp-seal                  ESP transform using SEAL cipher (160 bits)

esp-sha-hmac         ESP transform using HMAC-SHA auth

Note that right after you configure the transforms you get directed to configure further transform-specific parameters:

Router20(cfg-crypto-trans)#?

Crypto transform configuration commands:

default     Set a command to its defaults

exit          Exit from crypto transform configuration mode

mode       encapsulation mode (transport/tunnel)

no            Negate a command or set its defaults

STEP 4 Configure the IPsec Traffic Selectors

The traffic selectors are configured by defining extended access-lists.

The permit keyword causes all IP traffic that matches the specified

conditions to be protected by IPsec

access-list 121 permit tcp host 192.168.X.X host 192.168.X.X eq telnet

STEP 5 Configure the IKE Phase 2 (IPsec SA) Policy

This step sets up a crypto map which specifies all the necessary

parameters to negotiate the IPsec SA policy.  The following commands are required:

crypto map <crypto-map-name> <seq-num> ipsec-isakmp

match address <access-list-id>

set peer [IP address | hostname]

set transform-set <transform-set-name>

set security-association lifetime seconds <seconds>

set pfs [group1 | group 2]

Note:  Useful command is:

Router(config)#crypto map LINUXTELNET ?

<1-65535>        Sequence to insert into crypto map entry

client                 Specify client configuration settings

isakmp              Specify isakmp configuration settings

isakmp-profile   Specify isakmp profile to use

local-address    Interface to use for local address for this crypto map

redundancy       High availability options for this map

After entering a sequence number you can continue configuring:

Router(config)#crypto map LINUXTELNET 5 ?

ipsec-isakmp     IPSEC w/ISAKMP

ipsec-manual     IPSEC w/manual keying

<cr>

Note that more complex crypto maps can be configured.  But that is for more sophisticated labs 

Router20(config)#crypto map LINUXTELNET 5 ipsec-isakmp ?

dynamic     Enable dynamic crypto map support

profile         Enable crypto map as a crypto-profile

<cr>

So, when we hit <cr> we get a warning to remind us to configure a peer and valid access-list with this crypto map.

Router20(config)#crypto map LINUXTELNET 5 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

Router20(config-crypto-map)#?

Crypto Map configuration commands:

default            Set a command to its defaults

description     Description of the crypto map statement policy

exit                 Exit from crypto map configuration mode

match             Match values.

no                   Negate a command or set its defaults

qos                 Quality of Service related commands

reverse-route  Reverse Route Injection.

set                   Set values for encryption/decryption

STEP 6 Apply the IPsec Policy to an Interface

The configured crypto map is then applied to the appropriate

interface using the ‘crypto map crypto-map-name’ command.

Router(config)#interface FastEthernet 0/1

Router(config-if)#crypto map <crypto map name>

The resulting configuration should be something like this:

!

crypto isakmp policy 2

encr 3des

authentication pre-share

group 2

lifetime 28800

!

!

crypto ipsec transform-set CISCO2LINUX esp-3des esp-sha-hmac

mode transport

!

crypto map LINUXTELNET 5 ipsec-isakmp

! Incomplete

set peer 192.168.2.1

set transform-set CISCO2LINUX

set pfs group2

match address 121

!

interface FastEthernet0/1

description Link to Backbone Two

ip address 192.168.2.254 255.255.255.0

duplex auto

speed auto

crypto map LINUXTELNET

!

access-list 121 permit tcp host 192.168.2.254 host 192.168.2.1 eq telnet

Useful debugging commands:

Router20# debug crypto engine ?

accelerator  accelerator debug

error        Crypto Engine Errors

packet       Crypto Engine Packets

<cr>

Router# sh crypto ?

ca               Show certification authority policy

call             Show crypto call admission info

debug-condition  Debug Condition filters

dynamic-map      Crypto map templates

eli              Encryption Layer Interface

engine           Show crypto engine info

identity         Show crypto identity list

ipsec            Show IPSEC policy

isakmp           Show ISAKMP

key              Show long term public keys

map              Crypto maps

mib              Show Crypto-related MIB Parameters

optional         Optional Encryption Status

pki              Show PKI

session          Show crypto sessions (tunnels)

sockets          Secure Socket Information

Router# sh crypto isakmp ?

key      Show ISAKMP preshared keys

peers    Show ISAKMP peer structures

policy   Show ISAKMP protection suite policy

profile  Show ISAKMP profiles

sa       Show ISAKMP Security Associations

Router20# sh crypto ipsec ?

client                Show Client Status

policy                Show IPSEC client policies

profile               Show ipsec profile information

sa                    IPSEC SA table

security-association  Show parameters for IPSec security associations

transform-set         Crypto transform sets